# NPC – Unit 4 Assignment

Topic: Configure DHCP and Set Up a Network Using Cisco Packet Tracer

## Submitted By

Name: Kunal Kailas Kulkarni

PRN: 2125UCSM1078

Department: Cybersecurity

Course: Network Protocol Configuration (Fortinet)

Course Code: 24UCYPC201

Academic Year: 2026–27

### Objective

The objective of this mini-project is to configure a DHCP server in Cisco Packet Tracer, allow client PCs to obtain IP addresses automatically from it, and verify communication between the clients using the ping command.

### Software Used

Cisco Packet Tracer.

### Network Topology

The network consists of one server (Server0), one Cisco 2960 switch (Switch0), and three client PCs (PC0, PC1, and PC2). All devices are connected using copper straight-through cables. No router is used because all devices are in a single local subnet.

| From | Port | To | Port |
| --- | --- | --- | --- |
| Server0 | FastEthernet0 | Switch0 | FastEthernet0/1 |
| PC0 | FastEthernet0 | Switch0 | FastEthernet0/2 |
| PC1 | FastEthernet0 | Switch0 | FastEthernet0/3 |
| PC2 | FastEthernet0 | Switch0 | FastEthernet0/4 |

Figure 1: Network topology.

### IP Addressing

The server is configured with a static IP address, while the client PCs obtain their addresses automatically from the DHCP server.

| Device | IP Address | Subnet Mask | Method |
| --- | --- | --- | --- |
| Server0 | 192.168.2.2 | 255.255.255.0 | Static |
| PC0 | 192.168.2.100 | 255.255.255.0 | DHCP |
| PC1 | 192.168.2.101 | 255.255.255.0 | DHCP |
| PC2 | 192.168.2.102 | 255.255.255.0 | DHCP |

The default gateway and DNS server are set to 0.0.0.0 because this network has no router or separate DNS server.

### Procedure

Step 1: Create the Network Topology

One Server-PT device, one Cisco 2960 switch, and three PC-PT devices were placed in the Packet Tracer workspace. All devices were connected using copper straight-through cables according to the topology table. The link lights were checked to ensure that the connections were active.

### Step 2: Configure the Server's Static IP Address

On Server0, the Desktop → IP Configuration section was opened. The Static option was selected, and the following values were entered:

| Setting | Value |
| --- | --- |
| IPv4 Address | 192.168.2.2 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | Left blank |
| DNS Server | Left blank |

A static IP address was assigned to the server so that it could be accessed consistently by the client devices.

### Step 3: Configure the DHCP Service

On Server0, the Services → DHCP section was opened, and the DHCP service was switched On. A pool named LAN_POOL was created with the following configuration:

| Setting | Value |
| --- | --- |
| Pool Name | LAN_POOL |
| Default Gateway | 0.0.0.0 |
| DNS Server | 0.0.0.0 |

| Start IP Address | 192.168.2.100 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Maximum Number of Users | 50 |

Initially, the default pool named serverPool could not be removed or edited using the available controls, and PC0 received the address 192.168.2.1. To resolve the issue, serverPool was subsequently configured to use the same start address, 192.168.2.100, and a maximum of 50 users. The changes were saved, and the client PCs subsequently received the intended addresses.

Figure 2: DHCP server configuration.

### Step 4: Configure the Client PCs

Each client PC was configured to obtain its IP address automatically. The following steps were performed on PC0, PC1, and PC2:

- Open the PC device.

- Select Desktop → IP Configuration.

- Select DHCP.

- Wait for the message “DHCP request successful”.

- Verify the assigned IPv4 address and subnet mask. The client PCs received the following IP addresses:

| Device | Assigned IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.2.100 | 255.255.255.0 |
| PC1 | 192.168.2.101 | 255.255.255.0 |
| PC2 | 192.168.2.102 | 255.255.255.0 |

Figure 3: IP address assigned to PC0 using DHCP.

### Step 5: Verify the IP Configuration

The ipconfig command was executed in the Command Prompt on each client PC to verify the assigned IPv4 address and subnet mask.

The command used was: ipconfig

Each PC showed a unique IPv4 address in the 192.168.2.0/24 network with the subnet mask 255.255.255.0.

### Connectivity Testing

Connectivity was tested from PC0 using the ping command. PC0 sent ping requests to PC1 at 192.168.2.101 and PC2 at 192.168.2.102.

The following commands were executed:

ping 192.168.2.101

ping 192.168.2.102

These commands were used to verify communication between the client PCs within the local network.

### Ping Results

The connectivity test results are summarized below.

| Test | Result |
| --- | --- |
| PC0 receives an IP address using DHCP | Successful — 192.168.2.100 |
| PC1 receives an IP address using DHCP | Successful — 192.168.2.101 |
| PC2 receives an IP address using DHCP | Successful — 192.168.2.102 |
| PC0 pings PC1 at 192.168.2.101 | 4 sent, 4 received, 0% loss |
| PC0 pings PC2 at 192.168.2.102 | 4 sent, 4 received, 0% loss |

The ping tests confirmed that PC0 could communicate successfully with both PC1 and PC2. Both tests completed with zero packet loss.

Figure 4: Successful ping replies from PC0 to PC1 and PC2.

### Summary

A DHCP server was configured on Server0 with an address pool starting at 192.168.2.100. The three client PCs obtained their IP addresses automatically, and each received a unique address in the 192.168.2.0/24 subnet. The default pool, serverPool, was reconfigured during troubleshooting to match the intended address range. The ping tests confirmed that the PCs could communicate successfully over the local network.

### Result

The DHCP server was successfully configured in Cisco Packet Tracer. PC0, PC1, and PC2 received the IP addresses 192.168.2.100, 192.168.2.101, and 192.168.2.102, respectively. The ping tests from PC0 to PC1 and PC2 showed four successful replies each, with 0% packet loss.

### Conclusion

This mini-project demonstrated how DHCP automates IP address assignment in a network. Using a DHCP server eliminates the need to configure each client manually, helps prevent duplicate IP addresses, and simplifies network management. The successful ping results confirmed that the tested devices were correctly configured and could communicate within the same local subnet.

****